********************* Info ************************************

*1*  'General

Program CN2000 (CIEMAT/NIST calculations, year 2000)

----------------------------------------------------

A program to calculate the LSC efficiency of a nuclide vs. efficiency of the tracer H-3 (CIEMAT/NIST method).

History

Jun.99  CN99A,

Oct.99  CN99B,

Dec.99  CN99C,

Jan.00  CN2000A,

Sep.00  CN2000B finished

CN2000C started.

Author: E.Günther, PTB, section 6.11

Email address eberhard.guenther@ptb.de

Date: 01.10.2000

Language: Visual Basic for Windows 6.0

*2*          'Aims

Aims of the program

-------------------

The code is written for the efficiency calculation of

beta, beta-gamma,

EC, EC-gamma nuclides,

as well as for isomers and nuclides with a mixed decay.

It is

simple in the application,

ready-to-use for many radionuclides,

and therefore useful for the first C/N applications,

as well as for the efficiency calculation of nuclides with a more complex decay scheme.

The program includes

sets of about 80 radionuclides,

a preview to new EC radionuclides,

uncertainty calculations,

the use of different quench functions,

the use of different scintillators and kB values,

an output of the results as coefficients of a fit function of the nuclide efficiency vs. tracer efficiency,

this output as a table and a graphics display (HPGL or Excel),

a separate file with remarks on the actual calculation parameters and the general  calculation procedure.

*3*    'Start

----------------------------------------------------

How to start with this program

----------------------------------------------------

Please do not start with full speed. It can be frusty to read to much details.

Better is a start as simple as possible, e.g. with the following steps.

1. First calculation

Start the program, click the button 'Start' and wait what happens. On the screen you see a graph

of the efficiency of the nuclide vs. the efficiency of H-3. This graph and the coefficients of the fit

polynomial are the main output.

Answer the Word-question with 'y'. Then look into the word document 'OUTPUT [NuclideName].doc.

There you find the same graph at the end.

2. Choose the nuclide

Now change the input nuclide (here called the 'Job'): Start the program, click 'View of files',

'Control file...', 'CIENIST.job'. Write C14,-- on top of the file (not C-14 !). 'Save...', 'Start'.

After the calculation look in the word document 'OUTPUT C14.doc' for the constants of Eff(C14) vs. Eff(H3)

and calculate by foot the Eff(C14) for Eff(H3) = 50%. Compare this number with the output.

Will see the list of ready-to-use nuclide sets?: Start the program, 'View..', 'Nuclide data',

'Input data', 'List of nuclides'.

3. Change the calculation conditions

Start the program, 'View..', 'Control file...', 'CIENIST.ctl'. Edit the file part (3):

Output Modus (Min-1  Normal 0  Test 1)                 :, 1

...

TypQ TypCS nkB                                         :, KB, XCOM, 3

KB                                                     :, 50, 75, 100

'Save...', 'View of files', 'Control file...', 'CIENIST.job'. Write C14(3),--. 'Save...', 'Start',

for a change answer 'n', after the process ('End' is blinking) 'End'.

Start the program, 'View..', 'View..', 'Output files', '..Output.txt'. Look for the last 10 lines and find

two contributions to the uncertainty.

4. Now it is time for a first short look into the word document 'REPORT.doc'. It contains the special input and

calculation conditionscon as well as a comprimized description of the principles of the calculation. Check the

input and output section.

5. Next suggested steps: Cs134(0) (the same as Cs134 without the brackets), Cr51.

*4*          'Remarks

Remarks to this program

-----------------------

1. The code is based on the principles of the CIEMAT programs Effy, Vias, Cega2, Emi and KB.

2. The program is not yet ready developed. Errors are possible.

3. Uncertainty: Contributions to the uncertainty budget are calculated by use of fits.

4. Some more features are in preparation.

*5*    'What is new?

New in this version (in comparison to CN2000A)

==============================================

Part               New                                  Remarks

---------------------------------------------------------------------------------------------------------------------

General            Change from VB5 to VB6

English notation of numbers          Avoids errors by not-German versions of Windows, Excel, Word

Nuclide data input a. File CIENIST.job                  The number of the control data sets in 'CIENIST.ctl'.

b. File 'CIENIST.ctl'                More data sets, each set has a number

c. Numbers for the cascades          Each cascade has a number

d. Clear arrangement for cascades    e.g. B2 G2 G1 G5 for a cascade with beta and 3 gammas

Old input files are not more usable!

Control data       a. 2 Quench functions of A.Grau 2000 (As an option)

b. Cross sections                    From NIST program XCOM (As an option)

Fit output         Uncertainty contribution             Old fit file not more usable

of the KB uncertainty

Info               This info part

Trouble shooting   Test files

Word8 output       Faster                               Includes graphics

Planned for one of the next versions (?)

========================================

Part               Planned                              Remarks

---------------------------------------------------------------------------------------------------------------------------

EMI2               EC program part with KL1L2L3 model   The new program of A.Grau M. and A.Grau C.(2000) should be included.

But up to now it is not transparent, not treated for CN2000:

There are changes in quench treatment as well as in the ways (264 ways).

What is the influence of the quench function?

What is the influence of the input data uncertainties? On which way

can we get the input data? Many questions.

I am not sure to find time enough for this.

Results            Coefficients stored in a data base   I am not sure. Possibly it is not useful.

(Microsoft Access).

*6*    'Uncertainty

Uncertainty:

------------

In this part uncertainty contributions are calculated by variation of the input parameters.

*7*          'Test

Test calculations

-----------------

The program offers the following options:

Quench and interaction probability:

Click the button Test and start

Comparison of the output of this program with CIEMAT programs:

choose the following options, write them into the file CIENIST.ctl

and start efficiency calculation

Comparison with  Nuclide    QuenchType   kB  CrossSectionType  Scintillator  & String(77, "-") & vbCrLf

EFFY       C14         EFFY       -         -          Toluene (1,10,0)

BETA       C14         EFFY       -         -          Toluene (1,10,0)

CEGA2      Cr51        CIE      0.0075     CEGA        Toluene (1,10,0)

EMI        Cr51        CIE      0.0075     CEGA        Toluene (1,10,0)

*8*    'Trouble shooting

Trouble shooting:

-----------------

1. Many errors in the calculations come from errors in the input data.

Therefore an additional file \tmp\CTL.txt with the read input data is generated (have a look with the built-in viewer).

2. Special temporary data of the EC and gamma part of the program are in \tmp\EC_G.txt.

-------------------------------------------------------------------------------------------------------------------------

*9*

*18*          'Calculation results

File Output.txt

---------------

Full output of the results as an ASCII file.

The amount of output depends on the variable 'Output Modus' in the file CIENIST.ctl. The graphics is not included.

Files Output Nuclide.doc, e.g. Output C14.doc

------------------------

The same output of results as a Word8 file. With graphics.

*19*          'Calculation report

File Report.txt

---------------

This report contains remarks to the principles of calculation, descriptions of the used input files and parameters.

It is a report of the last efficiency calculation as an ASCII file.

File Report.doc (if the Word 8 option is chosen)

---------------

The same report as a Word file.

*20*          'Calculation results (Fit file)

Fit File

--------

An append file with the final results of all efficiency calculations.

It contains the coefficients of the fit polynomials Eff%(nuc) vs. Eff%(H3).

The nuclide efficiency table [Eff%(Nuclide) vs. fm value] was combined with the tracer efficiency table [Eff%(H3) vs. fm value].

The resulting table  [Eff%(Nuclide) vs. Eff%(H3)] was fitted with a polynomial, using the least-squares method.

The degree of the polynomial is chosen so that the residuals are not more than 0.01%.

The output fit file (as an appending file) includes in each data set

the nuclide name,

the half-life (in d),

the tracer,

the sample composition identification string (e.g. 5,15,1),

the quench data source,

the kB (in cm/MeV),

the region of calculation (tracer efficiency from % ... to %)

the date of the calculation,

the constants k(0) ... k(6) of the polynomial,

the reference efficiency (for a tracer efficiency of exactly 50 %),

and the uncertainty contribution of an 1 % tracer activity uncertainty.

*21*          'Graphics

Graphics: two different graphics are possible.

*22*          'HPGL

Graphic files nuc.hgl, e.g. C14.hgl

---------------------

with a HPGL graph of Eff%(nuc) vs. Eff%(H3).

This file can be imported directly by a word document.

*23*          'Excel

File CN00Calc.xls

-----------------

A new sheet with 2 graphs is added to the excel file:

a. A graph of Eff%(nuc) vs. Eff%(H3),

b. A graph with the contributions of the individual decay cascades in decay and measurement.

*24*

*********************************************************************************************

*33* 'Editor

Viewer / Editor:

----------------

Allows to edit files

*34*  'NucList

Nuclide list:

-------------

The list of nuclear data sets is written unsorted and sorted with increasing number.

It includes nuclides

which activity can be easily determined with the CIEMAT/NIST method                  (most beta and beta-gamma nuclides),

which activity can be determined with some trouble and larger uncertainties                           (most EC nuclides),

which activity can be not determined with the CIEMAT/NIST method                  (e.g. nuclides with long-lived levels),

After a query the results are stored in ...\CN2000B\OUT\NucList.txt.

*35*          'Preview

Preview:

--------

Gives a preview of the atomic data of an EC nuclide (pure EC), which is not included in the nuclide list.

All data are from fits. A preview of the efficiency is included.

An atomic data set for a first calculation is given.

After a query the results are stored in ...\CN2000B\OUT\Preview.txt.

***************************************************************************************************

Warning: These data are from fits.

They serve only for a first orientation and may differ from real values.

***************************************************************************************************

(to be continued with Atomic Data)

*36*           'Help programs

Help programs for getting the nuclear and atomic data for the input

--------------------------------------------------------------------

Program         for                      Remarks

--------------------------------------------------------------------------------------------------------------------------

Beta_Th.exe     Shape parameter          from Spin and Parity

EC_Th.exe       pK..pM                   only for allowed transitions

DecS.exe        Cascade probabilities    derived from the decay scheme

-------------------------------------------------------------------------------------------------------------------------

EC_Th.exe       wK, wL                   fluorescence yields

pKL, pKX                 X-rays: rel. probabilities

pKLL, pKLX, pKXY         K Auger electrons: rel. probabilities

pK, pL, pM               EC probabilities

-------------------------------------------------------------------------------------------------------------------------

KLL -AvEn.exe   EKL, EKX, EXY            X-rays: average energies

EKLL, EKLX, EKXY         K-Auger electrons: average energies

ELM                      L  X-rays: average energy

-------------------------------------------------------------------------------------------------------------------------

LMM -AvEn.exe   ELMM                     L-Auger: average energy

(incl. Coster Kronig. With difficulty.)

-------------------------------------------------------------------------------------------------------------------------